Washington State Department of Health

Office of Shellfish and Water Protection

(DATE)

(Example Effective Area) Recall Investigation Summary Report

SUMMARY:

Starting at approximately 6 PM on Tuesday (DATE), the operator of the (Example) wastewater treatment plant (WWTP) noted elevated color in the influent and elevated flows from storm inflow and infiltration (I/I).  These were occurring due to a rain storm.  The plant collected a fecal coliform effluent water sample at 3 p.m. on (DATE) and the result was 'too numerous to count' (TNTC).  On (DATE) at 11:20 AM the plant called the department and reported the high result.  The operator started injecting chlorine to supplement the normal UV disinfection upon getting the results.  The operator stated that all treatment hardware was in good working order and speculated that the TNTC result was due to the elevated color in the influent interfering with UV disinfection. He also reported that influent flows for (DATE) were about 50% above permitted maximum month design flows for the facility.  The insufficient disinfection impacted an estimated 1.2 million gallons of sewage in a 48-hr period.

Based on the fecal coliform sample result collected on (DATE) the (EXAMPLE AREA) growing area Conditionally Approved Area Management Plan was implemented and the department closed the Conditionally Approved area for five days from (DATE) until (DATE).  Growers were notified of the closure by 12:30 PM on (DATE).

All shellfish products harvested after 12:01AM on (DATE) were recalled.  Licensed companies involved were; (EXAMPLE COMPANIES INVOLVED with certification numbers).  The two (EXAMPLE COMPANIES) licensed companies did not harvest on that date.  (EXAMPLE COMPANY) shipped products to 19 customers in Washington State.  (EXAMPLE COMPANY) also shipped to 16 customers in 12 other receiving states.  A total of 13 states were involved in this recall.  All states were notified by email on (DATE) at 9:00 AM via email by the Department of Health.

The amount of Washington product recalled was 3,910 lbs of mussels, 190 lbs of clams and 370 dz oysters. The amount of out of state product recalled was 750 lbs of mussels, 925 lbs of clams and 1,110 dz oysters.  Total amount of product recalled was 4,660 lbs of mussels, 1,115 lbs of clams, and 1,480 dz oysters.  Out-of-state shellfish products shipped to 11 receiving states have been destroyed by the receiving states.  Shellfish products shipped to and located in Washington and Oregon have been picked up, returned to the dealer or destroyed on site.  Of the shellfish returned back to the dealer, the mussels were destroyed by the dealer at the local landfill and the returned oysters and clams were placed back into wet storage in (EXAMPLE COMPANY).

ILLNESS REPORT SUMMARY:

Not Applicable – no reported illnesses involved.

HARVEST SITE REVIEW:

The source of the product harvested was verified as licensed and certified by the Washington State Department of Health, as (EXAMPLE COMPANY).

RECALL OF PRODUCT:

Recall of product was initiated Thursday, (DATE) at 12:20 PM following notification by the department.  States involved in the recall are listed below:

US States Shellfish Shipments

| STATE | Dealer & Retail company receiving product | Species | Quantity | Product Disposition |
| --- | --- | --- | --- | --- |
| Alaska | (EXAMPLE COMPANY) | Mussels Clams | 25 lbs 175 lbs | (EXAMPLE COMPANY)picked up product in WA before shipping. |
| California | (EXAMPLE COMPANY) | Oysters | 60 dz 95 dz 10 dz | All product Destroyed |
| Florida | (EXAMPLE COMPANY) | Mussels Oysters | 110 lbs 10 dz | All product Destroyed |
| Illinois | (EXAMPLE COMPANY) | Clams Oysters | 25 lbs 100dz | All product Destroyed |
| Louisiana | (EXAMPLE COMPANY) | Clams | 100 lbs | All product Destroyed |
| Missouri | (EXAMPLE COMPANY) | Oysters | 20 dz | All product Destroyed |
| Nevada | (EXAMPLE COMPANY) | Mussels Clams Oysters Clams Oysters | 315 lbs 300 lbs 25 dz 250 lb 100 dz | All product Destroyed |
| New Jersey | (EXAMPLE COMPANY) | Clams Oysters | 10 lbs 60 dz | NJ notified (EXAMPLE COMPANY) that all product was Destroyed |
| Ohio | (EXAMPLE COMPANY) | Oysters | 110 dz | All product Destroyed |
| Oregon | (EXAMPLE COMPANY) | Mussels Oysters | 300 lbs 330 dz | (EXAMPLE COMPANY) Picked Up |
| Virginia | (EXAMPLE COMPANY) | Clams Oysters Clams Oysters | 30 lbs 40 dz 10 lbs 150 dz | VA notified DOH product was Destroyed; (35 dz Canadian product was not affected by Recall) Destroyed by shipping company |
| Wisconsin | (EXAMPLE COMPANY) | Clams | 25 lbs | All product Destroyed |
| Total | 16 | Mussels Clams Oysters | 750 lbs 925 lbs 1,110 dz | Mussels -100% Clams -100% Oysters -100% |

Washington State Shellfish Shipments

| STATE | Dealer & Retail company receiving product | Species | Quantity | Product Disposition |
| --- | --- | --- | --- | --- |
| Washington | (EXAMPLE COMPANY) | Mussels | 25 lbs | Picked Up-P/U 1/8/09 |
|  | (EXAMPLE COMPANY) | Mussels | 140 lbs | P/U - 1/13/09 |
|  | (EXAMPLE COMPANY) | Clams Oysters | 50 lbs 20 dz | P/U - 1/9/09 |
|  | (EXAMPLE COMPANY) | Mussels | 3,000 lbs | Destroyed at store |
|  | (EXAMPLE COMPANY) | Mussels | 70 lbs | P/U - 1/9/09 |
|  | (EXAMPLE COMPANY) | Mussels Clams Oysters | 5 lbs 5 lbs 3 dz | P/U - 1/8/09 |
|  | (EXAMPLE COMPANY) | Mussels | 10 lbs | P/U - 1/9/09 |
|  | (EXAMPLE COMPANY) | Clams Oysters | 10 lbs 5 dz | P/U - 1/9/09 |
|  | (EXAMPLE COMPANY) | Mussels Clams | 20 lbs 10 lbs | P/U - 1/9/09 |
|  | (EXAMPLE COMPANY) | Mussels Clams | 10 lbs 15 lbs | P/U - 1/9/09 |
|  | (EXAMPLE COMPANY) | Mussels Oysters | 60 lbs 60 dz | P/U - 1/9/09 |
|  | (EXAMPLE COMPANY) | Mussels Oysters | 200 lbs 70 dz | P/U - 1/9/09 |
|  | (EXAMPLE COMPANY) | Mussels Clams | 10 lbs 15 lbs | P/U - 1/9/09 |
|  | (EXAMPLE COMPANY) | Mussels Clams Oysters | 5 lbs 5 lbs 2 dz | P/U - 1/8/09 |
|  | (EXAMPLE COMPANY) | Mussels Oysters | 165 lbs 100 dz | P/U - 1/9/09 |
|  | (EXAMPLE COMPANY) | Mussels Oysters | 30 lbs 25 dz | P/U - 1/9/09 |
|  | (EXAMPLE COMPANY) | Mussels | 10 lbs | P/U - 1/9/09 |
|  | (EXAMPLE COMPANY) | Mussels Oysters | 50 lbs 25 dz | P/U - 1/9/09 |
|  | (EXAMPLE COMPANY) | Mussels Clams Oysters | 100 lbs 80 lbs 60 dz | P/U - 1/9/09 |
| Total |  | Mussels Clams Oysters | 3,910 lbs 190 lbs 370 dz | Mussels -100% Clams -100% Oysters -100% |

GROWING AREA INVESTIGATION:

There was no post-closure shoreline survey.

Water Quality Testing: The (EXAMPLE COMPANY) growing area CAAMP was implemented and the Conditionally Approved area was closed for five days from January 7 (when the TNTC sample was taken).  Growers were notified of the closure by 12:30 p.m. on (DATE).

Growing Area Classification Review:  Not applicable.

Growing Area Closure:  On (DATE) (EXAMPLE COMPANY) growing area was formally closed (e-mail listserve notice); starting (DATE)

Water Quality Results:  Not applicable.

SHELLFISH OPERATIONS/FACILITIES INVESTIGATION:  Not applicable.

BIOTOXIN MONITORING RESULTS:  Not Applicable.

CONCLUSION AND SUMMARY OF ACTIONS:

Recall of Product Confirmation:  Washington & out of state shellfish products recalled.

Reopening of (EXAMPLE) growing area:  The (EXAMPLE) growing area was re-opened on (DATE). The 5-Day closure was based on the CAAMP implemented for (EXAMPLE) Growing Area by the department.

Questions should be directed to (EXAMPLE) Point of Contacts.

Jessie J. DeLoach

Manager, Licensing and Certification